Recommended Reading - Rainfall Measurement Challenges - Steve Burges - June 2024

This is a guide to reading parts from the following 9 papers related to rainfall measurement and rainfall measurement under-catch.

Papers:

Pit and above ground rain gauge comp._Duchon, wrr_2001.pdf

Sieck_Burges_Steiner_Measurements of point rainfall_WRR 2007.pdf

Sieck_Burges_Steiner_Measurements of point rainfall_WRR 2007_correction.pdf

Rodda, J. C. (1967)_The rainfall measurement problem. Proc. IAHS Gen. Ass. Bern, IAHS Pub, (78), 215_231.pdf

Rainfall measurement revisited, J Rodda, H Dixon, Weather 65_5 2012.pdf

Plynlimon_Celebrating 50 years of UK, Research in Hydrology, John Rodda, AWRA Impact 2018.pdf

Kampf_Burges_Quantifying the water balance in a planar hillslope plot_J of Hydrology_2010.pdf

Kampf_Burges et al, The Case for an Open Water Balance Reenvisioning Network Design and Data Analysis for a Complex, Uncertain World, wrr 2020.pdf

Invention of Meteorological Instruments_Rain Gauges_Middleton_1969.pdf

How much rain does a rain gage gage, E L Neff, J Hydrology 1977.pdf

A. Accuracy of Hydrologic Measurements:

Kampf_Burges et al, 2020 – Read Section 2 for uncertainties and their propagation.

B. Rainfall Measurement:

The single most important quantity for snow absent catchments to measure accurately is liquid precipitation.  The best way to measure this is using a “pit gauge” or nest of such gauges.

See page 142-143, “Invention of Meteorological Instruments_Rain Gauges_Middleton_1969” for descriptive details of the 1842 “pit gauge” developed by Thomas Stevenson.  His work led to the adoption of a 1 foot high UK standard rain gauge.

The influence of wind on rain under-catch:

1. Rodda, J. C. (1967)_The rainfall measurement problem. Proc. IAHS Gen. Ass. Bern, IAHS Pub, (78), 215_231 See = Fig 5. For summary of under-catch of standard gauge relative to a pit gauge at Wallington UK.

2. Rainfall measurement revisited, J Rodda, H Dixon, Weather 65_5 2012.  See Fig 4, Fig 5, and Table 2 for details of measurements at 43 UK sites.

3. How much rain does a rain gage gage, E L Neff, J Hydrology 1977.  See details for empirical (regression) corrections of wind affected gauges at six sites (Table 1)

See Discussion (Page 218) – This should be taught to all hydrologists (extract below with emphasis by Steve Burges).

Neff (1967) recommended that those involved in measuring precipitation or using precipitation records develop an awareness of the limitations of these data so that no more confidence is placed in the records than is justified by the measurement techniques. Records, once collected and published, often gain an aura of respectability and precision that is beyond tolerances that can legitimately be assigned to them. Rain-gage records should not be used without considering the effects of potential error because errors become more important as we proceed from empirical studies in which the precipitation value may be only an index to more basic studies which require absolute values. A 5 or 10% error may be relatively unimportant in data from a rain gage that is a part of the climatological network whose purpose is to provide index values for general climatic characterization of the area represented by the gage. However, a 5 or 10% error may be very important in a hydrological network, whose purpose is to provide quantitative precipitation input to an intensive hydrologic investigation. Then, the rain-gage error may be larger than other components being studied in the hydrologic cycle.

4. detailed measurement and documentation of rain under-catch

Pit and above ground rain gauge comp._Duchon, wrr_2001.  See Section 7 and particularly “Section 2”

5.  Causes of rain under-catch in a wind exposed gauge

A complete treatment, combined with numerous measurements, is given in “Sieck_Burges_Steiner_Measurements of point rainfall_WRR 2007”.

See 3. Data Quality Control and Rain Gauge Calibration

See Fig 7 – difference between sARS and sTB3.  We now know that the TB3 siphon mechanism causes under-catch in strong wind situations.

Section 4.1 (see also paragraphs 56, 60, and 63)

C. Consequences of rain under catch bias on the water balance

Kampf_Burges_Quantifying the water balance in a planar hillslope plot_J of Hydrology_2010

See Fig 8 and associated discussion.

For problems of the TB3 gauge – see page 195 – Right column second paragraph (reproduced below):

“There is an additional problem of under-catch of the TB3 gauges that we are still documenting. These gauges are designed to meet the Bureau of Meteorology, Australia, standards for rugged instruments that will provide accurate measurement of the rain that passes through the funnel to within ±3% for rain rates that range from 50 to 500 mm h_1. Most designers of tipping bucket rain gauges (including Hydrological Services) elect to meet this standard by using a siphon that collects the flow from the funnel (Fig. 2c). When the siphon is full it discharges at a constant rate to the tipping buckets. When empty, an air path relieves funnel pressures from fluctuating wind. When the siphon has collected some water but is well below full, there is no longer an air path through the funnel. Consequently, during wind gusts (even for pit gauges) hydrometeors are blown out of the funnel domain.  We have documented under-catch on gusty days of up to 15% relative to adjacent pit gauges that maintain air paths through their funnels.”

See also:

U.S. Climate Reference Network after One Decade of Operations_Howard J. Diamond_BAMS 2013

The background for establishing this network is given starting in paragraph 3.  Note the complete absence of any pit rain gauge in Figure 1.