Proposal for changes to Ignition: Autoignition:

Move the following from the page (surftemperdetail.html) titled: 
Surface Temperatures
Detailed Information and References
Delete:
“*add graphs here”
Move the following to a new page: 

(Add main title as immediately below:)
Autoignition
Ignition Is a Probabilistic Function of Surface Temperature
Hot surface ignition of fluids involves a complicated function of factors, surface temperature being just one of them. Even at a constant surface temperature in the same engine, it is possible to have ignition at some times and not others. For example, if wind increases, the mixture may become too lean such that ignition is no longer supported. Some tests have shown that ignition of an underhood fluid in a controlled laboratory environment will have three ranges of surface temperature [1,2,3]: 

1. No ignition is observed

2. Ignition occurs some of the time (becoming increasingly more likely as the temperature increases)

3. Ignition occurs virtually all the time (under the same conditions)

The graph (below) showing this trend is included from one such study. For engine oil ignition, the probability of ignition below about 570 degrees F (300 degrees C) is near zero; it increases as shown to about 635 degrees F (335 degrees C), and ignition occurs predictably above that temperature [1].

[image: image1.jpg]Probability of ignition

1.0

* Oserved ) - Unused Motor OF
Predicted K1) - Unused Motor OF
Predicted 95% LCL - Unused Motor OF

- Predicled 95% UCL - Unused Motor O
P — e

310 315 320 325 330 33 340 345
Temperature (C)




To understand the potential for autoignition, compare the autoignition temperatures of the fluid in vehicle environments to the hypothesized surface temperature and determine which of the three regions of ignition probability is likely to be appropriate to the case being investigated.

 

References
1. Santrock, J. “Flammability Properties of Engine Compartment Fluids Other than Gasoline,” NHTSA 98-3588-193, 1998.

2. LaPointe, R., et al., "Hot Surface Ignition of Gasoline on Engine Materials," SAE 2006-01-1013, 2006.

3. Davis, S., et al., "Hot Surface Ignition of Flammable and Combustible Liquids," SAE 2006-01-1014, 2006

Hyperlink settings on page:

Back – to “Surface Temperatures: Underhood”
Next:
to Video page below
Navigation bar: Igniton: Autoignition
Related change to page (surftemperdetail.html) titled:
Surface Temperatures
Detailed Information and References
Hyperlink settings on page:
Next should be to new page above
Back should be unchanged
Navigation bar unchanged
Related change to page (surftemper.html) titled:

Surface Temperatures: Underhood

Hyperlink settings on page:
Next should be to new page above: “Ignition is a probabilistic function…”
Back should be unchanged
Navigation bar unchanged

Add new page for video
Page title: 

Autoignition
Videos of Ignition Testing
The previous pages in this section have introduced the concept of autoignition (hot surface ignition) in vehicle fires and provided some information about the surface temperatures available in vehicle environments under various operating conditions. The page showing the probabilistic nature of the ignition events also demonstrates the complicated nature of hot surface ignition. That complexity is also apparent in videos of hot surface ignition tests. 
The high speed video (immediately below) shows a controlled stream of 0.25ml diesel fuel contacting a hot surface maintained at a temperature of 750 ⁰C. The fuel vaporizes on the hot surface and vapors are somewhat contained by the recessed fixture; ignition is delayed until a combustible mixture is formed over an area of the plate. Combustion continues until all liquid is vaporized. Note: The video is being played back in quarter speed.

Add video: Fire_Diesel_Ex.mov

Video courtesy of Exponent, Inc.

The next video shows a similarly controlled stream of bio-diesel (B100) contacting the hot surface maintained at 450 ⁰C in the same test fixture. This high speed video, shot at 500 frames per second and played at 1/8th speed, again shows the ignition delay and more clearly the spread of combustion through the vapor cloud. Once the hot surface ignites the vapor in one area, piloted ignition spreads rapidly through the rest of the vapor. It is easy to see how introduction of air flow could disrupt the ignition during the delay.

Add video: Fire_B100_Ex.mov

Video courtesy of Exponent, Inc.

More information about the testing can be found in references 1 and 2.

The final video was taken by an infra-red video camera and also involved hot surface ignition of bio-diesel on a steel surface. In this test, a single drop is released onto the surface; the temperature distribution of the surface can be interpreted by comparing the color of the image to the legend. Again the liquid vaporizes on the surface and ignition is delayed until an appropriate mixture contacts an area of the surface of sufficient temperature. The video also shows the motion of the droplet on the flat surface as it vaporizes.
Add video: Hot Surface Ignition.wmv

Video courtesy of Fire Research Group, University of Waterloo.

More information about the testing can be found in reference 3.
References
1. Davis, S., Chavez, D., and Kyotma, H., “Hot Surface Ignition of Flammable and Combustible Liquids,” SAE 2006-01-1014, 2006.

2. Somandepalli, V., Kelly, S., and Davis, S., “Hot Surface Ignition of Ethanol-blended Fuels and Biodiesel,” SAE 2008-01-0402, 2008.

3. Byers, K., Epling, W., Cheuk, F., Kheireldin, M., and Weckman, B., “Evaluation of Automobile Fluid Ignition on Hot Surfaces,” SAE 2007-01-1394, 2007.

Hyperlink settings:

Next: to “Autoignition Characteristics of Fuels” (fuelref.html)
Back: to the new page, above, “Ignition is a probabilistic function…”
Add line to Navigation Bar: Ignition: Autoignition: Test Video (if it fits) – if not, just “Test”
