Computing randoms with SimSET

Author: Robert Harrison

Date last revised: July 2006

# Overview

SimSET has never been able to directly simulate randoms and other effects dependent on the relative time of decays because it produces decays for one voxel at a time. With the availability of inexpensive100+ gigabyte disc drives and faster computers, it should now be feasible to simulate randoms, triples, and deadtime by sorting list mode SimSET data by detection time and then filtering the results with appropriate algorithms.

Unfortunately this will require turning off all the importance sampling and generating both decays and photons with weights exactly 1. It will probably be necessary for people to do shorter simulations to estimate these effects than they use for simulation of true and scatter events. (An alternate possibility is to create a list-mode file using importance sampling, then bootstrap resample a list-mode data set of weight=1 decays and photons from the importance sampled list, assigning each resampled decay a random decay-time. The resampled list would then be used as input to the sorting routine.)

This document summarizes the changes necessary to do so.

# Required changes

- Every decay will be time-stamped with a decay-time.

  - This will be done using a ‘long long’ data type. A random integer, R, between 0 and (scan length * 1E12) will be assigned, giving the decay time in picoseconds after the start of the scan.

- SimSET needs to use a better algorithm than currently used to generate the random number: our current generator produces an integer between 1 and ~2E9, which we then divide by the maximum random number to generate a number between 0 and 1. This would not come close to generating decay times with 1 nanosecond (much less 1 picosecond) resolution even for the default 60 second length_of_scan.

- When generating PET data, the user must be able to set a flag to let single photons pass through the simulation. (Otherwise they are not passed on from module-to-module for efficiency reasons.)

- The user must be able to have SimSET decide the number of decays to simulate so that all decays/photons can have weight 1.

  - Setting the number of decays to simulate to 0 will be the flag for this.

- A new header field will indicate whether the data set was generated with time-stamps or not. A field should also be added for the time window for the randoms processing (see below).

  - This will require a new header version number. (Not true – the current header standard is expandable.)

  - The routines for reading in and writing out headers will need to be modified to handle this field.

  - Another change I am considering is to use 64-bit integers (on systems that support them) to represent the number of decays and the counting variables that are closely related to it. Several users have requested the ability to simulate 10s or even 100s of billions of decays, which is not possible with the 32-bit integers we currently use. This is not directly related to the changes detailed here, but it would be good if these changes were made in the same release version to minimize the number of data, header, and parameter file formats out in the wider world.

- The functions for reading and writing list mode files will be modified to handle the decay-time field in the decay when it is present.

  - The function will need to be able to read in decays whether they have the decay time or not.

  - Perhaps, instead, we could write a utility to add decay-time to list-mode files from the previous versions. (Taken care of in oldReadEvent().)

- A sorting routine will be written for SimSET list-mode data. It will sort the decays by decay time, creating a new list-mode data set that has the same format as regular SimSET list-mode data.

  - The routine will work with standard list-mode files, but not necessarily with custom list-mode files (i.e., if whoever is programming it cares enough about it, they can make a custom list-mode option).

  - The routine should be implement as a SimSET utility (i.e. like makeindexfile).

  - Steven Gillispie points out that disk I/O could be a major part of the sort time and suggests MergeSort.

  - The functions LbHdrWrite and LbHdrRead can be used to read in/write out the header.

  - PhoHFileWriteDetections can be used to write out the list-mode file decay-by-decay.

  - phgrdhstStandard shows how to use readEvent (both functions are in bin.phg.c) to read in a decay.

- A randoms/triples/deadtime routine will be written for decay-time sorted SimSET list-mode data. It will read in the list-mode data and write out regular SimSET list-mode data. The output list mode data does not need to be time sorted.

  - The routine will work with standard list-mode files, but not necessarily with custom list-mode files (i.e., if whoever is programming it cares enough about it, they can make a custom list-mode option).

  - The input file needs to be time-sorted. (An error should be generated if not).

  - The routine should be implemented as a SimSET utility (i.e. like makeindexfile).

  - The functions LbHdrWrite and LbHdrRead can be used to read in/write out the header.

  - PhoHFileWriteDetections can be used to write out the list-mode file decay-by-decay.

  - phgrdhstStandard shows how to use readEvent (both functions are in bin.phg.c) to read in a decay.

  - The routine will need a parameter that gives the size of the time window within which randoms, triples, and deadtime will be considered. This will be a new header field. As each decay is read in, the detection time for each of its photons will be computed using the decay-time and the photon’s travel_distance. Each photon will be compared to all the photons within the time window around it, before or after.

  - If two photons from separate decays are written to the output list-mode file to create a random, a pseudo-decay will be created for the two photons. Decays will have a field marking them as ‘true’ or ‘pseudo’. (Is this enough? Is there some other type of event we should consider? How about in SPECT? How about a triple detected as an event, should we have the possibility of a ‘triple’ decay type?)

  - I propose the following randoms/deadtime/triples algorithms as defaults. They are quite simple, I am interested in feedback on whether they are useful.

    - First of all, we ignore deadtime.

    - If three or more photons hit within the time window, we discard them all. (By distinct I mean that only one blue and/or one pink photon from each decay can be counted as part of the three.) Is there any reason to have a different time window for this than for randoms?

    - If two decays each with one photon remain within the time window, the two photons will be written out in a single pseudo-decay. The pseudo-decay will be given a decayWeight = eventWeight = 1. The decay location will be set to (0,0,0), though, in truth, pseudo-decays have no location. If one decay remains and it contains both pink and blue photons, then the decay and its photons will be written to the output list without modification.

    - In addition to these algorithms (or whatever algorithms we decide on) we should include user functions for deadtime, triples, and randoms where the user can change the action of the algorithms.

- The PET binning function will be modified so that it can produce separate histograms of non-random coincidences and random coincidences (and triple coincidences?) (in combination with all the other possibilities currently there).